|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Portland State University |  |  |
|  | Facilities Services |  |  |
|  | ## Operating a Bench Grinder |  |  |
|  | Task | Hazards | Controls |
|  | - Check position of tool rests and condition of wheels. | Loose or poorly adjusted tool rests create pinching hazards. | Adjust and tighten tool rests. |
|  |  | Cracked or broken wheels create projectile hazards. | Confirm wheels are not cracked or broken. |
|  | 2. Turn on grinder. | Broken pieces of wheels striking operator. | Stand off to the side of the grinder when turning it on. |
|  | 3. Grind object. | Abrasion and burns to fingers and hands | Wear leather gloves. Keep fingers and hands away from wheels. |
|  |  | Flying sparks and debris | Wear safety glasses or face shield. Keep flammable materials away from grinding operation. |
|  |  | Pinch to hand. | Wear leather gloves. Keep fingers away from pinch points. |
|  | Required Training: 1. Operation of grinder | Required Personal Protective Equipment (PPE) - Leather gloves - Safety glasses - Face shield |  |
| Other Information: |  |  |  |
| Contributors: | Reviewed JKM |  |  |
| Created: | July 2006 |  |  |