|  | JOB HAZARD ANALYSIS Facilities & Property Management Operating a Chainsaw |  |  |
| --- | --- | --- | --- |
|  | Task | Hazards | Controls |
|  | 1. Pre-work Briefing/Tailgate Safety Session | - None | - Brief employees on work assignment and objectives - Ensure required PPE is being utilized - Review JHA - Brief employees on the plan that would be executed in the event of a serious employee illness/injury that would require medical evacuation |
|  | 2. Prepare work area | - Tripping hazards - Injuries to bystanders and co-workers | - Clear brush and debris, visually examine area to establish secure footing - Warn people in or near area by utilizing barricades, cones, or caution tape and spotters to keep bystanders a safe distance from job; consider the shape and lean of tree; consider wind force; plan escape route from work area in case of danger, |
|  | 3. Start saw | - Cutting injuries to hand, feet, body parts - Flying debris entering eyes - Hearing loss | - Start saw on ground with foot on rear handle, left hand firmly gripping saw grip, right hand pulling starter cord. Maintain secure footing. Wear cut-resistant gloves, hard hat, cut-resistant chaps, hard-toed sturdy work boots, and safety vest. Make sure co-workers are clear of saw. - Wear safety glasses with side protection, face shield. - Wear ear plugs or ear muffs. |
|  | 4. Cutting wood | - Cutting injuries to hand, feet, body parts - Log or limb rolling into body - Kickback, blade backing away from wood - Flying debris entering eyes - Muscle strain, fatigue - Injuries to bystanders and co-workers - Hearing loss - Heat stress | - Maintain proper stance and hold; keep left arm and elbow straight, two hands on saw, body to the left of saw; maintain secure footing; never cut above shoulder level; cut with the underside of saw blade as much as possible. Stop engine for all cleaning, refueling, adjustments, and repair of saw. - Stand upslope so cut pieces roll away from feet, legs, and body. - If rolling is anticipated, secure area downslope and secure large pieces. - Make sure blade is moving before making contact with material; use wedge when necessary to keep blade from binding; make sure solid objects like rocks or concrete do not contact blade; use 90-degree notch and back-cut techniques on standing trees >5 inches in diameter. - Wear safety glasses with side protection, face shield. - Utilize proper body position; stretch, take frequent breaks to maintain alertness; stay hydrated. - Maintain awareness of work areas; check barricades; do not approach operators; shout warnings to co-workers and bystanders. - Wear ear plugs or ear muffs. - Take more frequent breaks during warmer weather. - Drink plenty of fluids. |
|  | 5. Carrying saw | - Cutting hands, feet, body | - Idle engine when carrying less than 100 feet; stop engine when carrying saw more than 100 feet or at any time you exit the secured area; never carry saw over shoulder. |
|  | 6. Fueling chainsaw | - Burns, Possible fire | - Allow chain saw to cool for at least 5 minutes before refueling. - Fill the tank on bare ground or other noncombustible surface. - Immediately clean up spilled fuel/use a funnel. - Refuel outdoors and at least 20 feet from an open flame or other sources of ignition. - Start the saw at least 10 feet from the fueling area. |
|  | Required Training: - Operation of chainsaw - First aid, plus training in use of chainsaw emergency first aid kit Required Equipment: - Chainsaw emergency first aid kit | Required Personal Protective Equipment (PPE) Hard hat *Spotters and ground crew are required to wear all PPE except chaps. - Ear plugs - Safety glasses - Face shield - Cut Resistant Gloves - Cut Resistant Chaps - Hard-toed work boots - High visibility vest - Long sleeved shirt and pants |  |
| Other Information: Contributors: Created: | Karen Barnack, EHS; Leslie Walters, FPM Landscape Supervisor January 2020 |  |  |